MICHIGAN

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION

FOR

SITE FURNISHINGS REMOVE, SALVAGE, AND REINSTALL

NOR:DWM	1 of 2	APPR:KGQ:NJM:08-03-22

a.	Description.  This work consists of removing, salvaging, and reinstalling the salvaged site furnishings within the project as specified by the Engineer, which includes trash receptacles and benches.  This work includes any necessary excavation, backfilling, and disposal of unsuitable materials required for a complete installation.

b.	Materials.  Furnish the anchors bolts, nuts, washers, fasteners, and all other hardware required for installation per subsection 908.09.C of the Standard Specifications for Construction.  Existing materials may be reused if approved by the Engineer.  Ensure the materials and workmanship are in accordance with sections 901, 902, 903, and 1004 of the Standard Specifications for Construction.

1.	Trash Receptacles.

A.	Mount the trash receptacle per the manufacturer’s recommendations, or as directed by the Engineer.

B.	Ensure all anchoring bolts, nuts, washers, and all other hardware used for installation are galvanized.

2.	Benches.

A.	Use materials in the reinstallation of the bench in accordance with the details included on the plans.  Surface-mount the bench per the manufacturer’s recommendations, or as directed by the Engineer.

B.	Ensure all anchoring bolts, nuts, washers, and all other hardware for used for installation are galvanized.

c.	Construction.  Remove and salvage benches and trash receptacles including all associated hardware in accordance with subsection 204.03 of the Standard Specifications for Construction.  Replace any hardware damaged during removal.  Install and anchor the site furnishings in accordance with the manufacturer’s specifications and as directed by the Engineer.

1.	Install the trash receptacle level and square in a true, flat plane to prevent rocking.

2.	Install the bench level and square in a true, flat plane to prevent rocking.  Ensure the seat height is 17 inches or as approved by the Engineer.

d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay items:

Pay Item	Pay Unit

Trash Receptacle, Rem, Salv, Reinstall	Each

Bench, Rem, Salv, Reinstall	Each

1.	Trash Receptacle, Rem, Salv, Reinstall includes the removal, salvaging and installation of an existing trash receptacle.

2.	Bench, Rem, Salv, Reinstall includes the removal, salvaging and installation of an existing bench.