ATTACHMENT A - MUST DESCRIBE THE PROJECT SPECIFIC AREAS AND/OR BUILDINGS TO BE FILMED.  Delete these instructions and submit to the Special Provision mail box for review when Attachment A is completed.

MICHIGAN

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION

FOR

AUDIO-VISUAL FILMING

SOURCE CODE	1 of 3	APPR:DMG:DBP:01-10-19

a.	Description.  This work consists of providing a film (DVD) record of physical, structural and aesthetic conditions of the area described in Attachment A of this special provision as it exists prior to the beginning of any construction activity.  Ensure the film is of professional quality and provides a clear and accurate visual record of existing conditions.

Complete filming, under the supervision of the Engineer, before any construction activity is started.  Furnish the completed DVD to the Engineer at least 7 calendar days prior to placement of any materials or equipment in the construction area.  Re-film any portion of the DVD determined by the Engineer to be unacceptable for the documentation of existing conditions prior to the start of any construction activity at no cost to the Department.

b.	Audio-Video Production.  Provide the name of the video recording services company to the Engineer at least 5 work days prior to the start of filming.

Use DVD format and equipment which allows audio and video information to be recorded.  Film in the general direction of vehicular travel and do not exceed 45 feet per minute (approximately 0.5 miles per hour).  Control pan and zoom rates to ensure playback clarity of the subject matter being filmed.

Use audio and video cues to identify location, relative to project limits and landmarks, at intervals of not more than 100 feet along the filming route.  Provide audio commentary as necessary during filming to describe streets, buildings, landmarks and other details which will enhance the documentation of existing conditions.

Conduct filming during a time of good visibility and not during periods of precipitation, or when snow, leaves or other natural debris obstruct the area being filmed.

Use video equipment with date/time stamp and digital annotation capabilities.  The final video recording must display the date (month, day and year) and the time (hours, minutes and seconds).  This transparent information is to appear on the upper left-hand corner of the frame.

The project station numbers must appear in the lower half of the frame.  This stationing must use MDOT standard engineering symbols (i.e., 3+50).  If there is no project stationing in an area being filmed, assign assumed stationing to each street or other discrete area being filmed.  Start the assumed stationing at 0+00 and increase from west to east or from south to north.  Include periodic, transparent alpha/numeric information below the station numbers consisting of the name of the project, name of area shown, direction of travel, viewing direction, etc.

c.	Area to be filmed.  Film all existing driveways, sidewalks, fences, trees, shrubbery and other structures and landscaping located up to 50 feet outside of the proposed right-of-way line shown on the plans and within the limits described in Attachment A.  The video coverage must include the interior and exterior of all buildings listed in Attachment A.

d.	Measurement and Payment.  The completed work, as described, will be measured as a lump sum and paid for at the contract price using the following pay item:

Pay Item	Pay Unit

Audio-Visual Filming	Lump Sum

Audio-Visual Filming includes all labor, equipment and materials required to perform the filming and to provide the finished DVD(s) to the Engineer.

ATTACHMENT A

LIMITS OF AUDIO-VISUAL FILMING