MICHIGAN

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION

FOR

POST, STEEL, 3 POUND, MODIFIED

MTP:JTK	1 of 1	APPR:MWB:AJU:05-06-22

a.	Description.  This work consists of installing painted three-pound steel sign supports for the relocation of existing City of Midland wayfinding signs at the locations shown on the plans.

b.	Materials.  Furnish sign post materials in accordance with subsection 810.02 of the Standard Specifications for Construction.  Furnish red color acrylic polyurethane paint to match City of Midland exterior signage and display hardware standard (Matthew’s Paint Company specification number MP 10256).  Gloss finish of paint specified must be 60 degrees or 29.8 on a 60-degree glossmeter.

c.	Construction.  Complete post installation in accordance with section 810 of the Standard Specifications for Construction.  Following installation, paint posts with acrylic polyurethane paint.

d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay item:

Pay Item	Pay Unit

Post, Steel, 3 pound, Modified	Each

Reinstalling salvaged existing wayfinding sign panels will be measured and paid for separately.