MICHIGAN

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION

FOR

TEMPORARY PEDESTRIAN PATH

OFS:RAL	1 of 2	APPR:DMG:CAL:05-01-18

a.	Description.  This work consists of furnishing, installing, maintaining, and removing a temporary pedestrian path as identified in the proposal or on the plans.  Temporary pedestrian paths, or segments thereof, will be repaired or replaced as directed by the Engineer.

b.	Materials.  Provide materials to construct a temporary pedestrian path in accordance with the contract, the Public Right of Way Accessibility Guidelines (PROWAG), the MMUTCD, as directed by the Engineer, and the following requirements:

1.	Ensure the materials used to construct the temporary pedestrian path yields a continuous hard surface that is firm, stable and skid resistant.  Ensure the path does not warp,  buckle or otherwise become uneven, and materials support the weight of pedestrians as well as motorized scooters and wheelchairs.  Suitable materials to construct the path include asphalt materials, Oriented Strand Board (OSB), plywood, dimensional lumber, reclaimed, or other as approved by the Engineer.  Compacted soils, aggregate and sand are prohibited.

2.	If asphalt materials are not used to construct the path, provide an antiskid coating, or surface treatment as directed by the Engineer.

c.	Construction. Construct the temporary pedestrian path in accordance with PROWAG, the MMUTCD, the contract, the direction of the Engineer, and the following:

1.	The useable surface of the path must be a minimum of 48 inches wide, additional width may be provided to preclude the use of Temporary Pedestrian Passing Spaces (paid for separately).  A minimum width of 60 inches is required if Temporary Pedestrian Passing Spaces are not provided as part of the temporary facility.  The maximum cross slope for the path is 2 percent.  The path, including transitions to the adjacent surface at both ends, must be free of vertical discontinuities greater than 1/4 inch.  Eliminate any vertical discontinuities greater than 1/4 inch up to 1/2 inch or bevel with a slope not steeper than 1:2.  If a vertical discontinuity greater than 1/2 inch or a running slope greater than 1:20 occurs on the project, a Temporary Pedestrian Ramp (paid for separately) is required.

A.	Ensure an anti-skid surface treatment is applied to the surface of the path, if not constructed with asphalt materials, as directed by the Engineer.

B.	If the surface of the path is constructed from OSB, plywood, or dimensional lumber securely connect all sections with appropriate fasteners to ensure a continuous, uniform and flat surface.

2.	Ensure all debris and construction materials is cleared from the path throughout its use.  Ensure snow and ice is removed; the use of an approved de-icing agent may be required.

3.	Repair or replace the path, or segments thereof, if it becomes uneven, unstable, or displaces due to weather events, construction activities, or other causes as directed by the Engineer.

4.	Following the use of the temporary path, the Contractor must remove and dispose all materials used to construct the path, and restore the area as directed by the Engineer.

d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay item:

Pay Item	Pay Unit

Pedestrian Path, Temp	Foot

Pedestrian Path, Temp will be measured along the centerline of the path.  Pedestrian Path, Temp includes all costs related to installation, maintenance, restoration, and removal of the path and disposal of all associated materials throughout the life of the contract.