- MICHIGAN

- DEPARTMENT OF TRANSPORTATION

- SPECIAL PROVISION

- FOR

- Concrete Barrier-Mounted Temporary Sign System

- OFS:RAL	1 of 2	APPR:CRB:CT:12-05-14

a.	Description.  This work consists of furnishing, installing, maintaining, relocating, and removing a temporary sign system designed to attach to temporary or permanent concrete barrier as identified in the proposal, on the plans, or as directed by the Engineer.  Damaged sign systems will be replaced as directed by the Engineer.

- b.	Materials.  Provide a sign system that meets the following requirements:

- 1.	Construct the sign system from materials that will not be susceptible to corrosion or deterioration due to the effects of weather, or road de-icing agents throughout the duration of installation.

- 2.	Ensure the sign system has the capacity to display a roll-up or rigid sign up to 48 inches wide by 60 inches high.  Ensure the sign system and design meets all of the applicable requirements from subsections 812.03.D and 922.02 of Standard Specifications for Construction.

- 3.	Ensure the sign system securely mounts to the barrier section in a manner that displays the sign clearly above the barrier.  Anchoring mechanisms must operate such that when engaged, they do not irreparably damage the barrier section.

- Ensure the sign system is capable of being properly attached, as determined by the Engineer, to concrete barrier, temporary or permanent, with top width as narrow as 5 inches or as wide as 15 inches.

4.	Select one of the following Concrete Barrier-Mounted Temporary Sign Systems:

A.	Median Barrier Bracket, manufactured by MDI Worldwide., 38271 W Twelve Mile Rd, Farmington Hills, MI, 48331, (800) 521-6776;

B.	Barrier Buster, manufactured by Traffix Devices Inc., 160 Avenida La Pata, San Clemente, CA, 92673, (949)361-5663;

C.	Other Concrete Barrier-Mounted Temporary Sign Systems of equal specifications may be utilized if approved by the Engineer.  Obtain the Engineer’s approval before installing an alternate device.

c.	Construction.  Deploy the concrete barrier-mounted sign system in accordance with the manufacturer’s recommendations, and the following requirements:

1.	Install the sign system at locations shown on the plans or as directed by the Engineer such that the message displayed is easily visible to motorists and not obscured by or in conflict with other signage.  Make adjustments to the position of the sign system as directed by the Engineer.

2.	Ensure any deformation or damage to a Department-owned concrete barrier section which occurred due to installation or use of this sign system is repaired as directed by the Engineer at the time of removal.

- d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay item:

- Pay Item	Pay Unit

- Conc Barrier-Mounted Temp Sign System, Furn	Each

- Conc Barrier-Mounted Temp Sign System, Oper	Each

1.	Conc Barrier-Mounted Temp Sign System, Furn, includes all labor, equipment, and materials to furnish and install posts, bases, and mounting hardware at the locations shown on the plans and replacement of sign systems damaged by vehicular traffic other than the Contractor’s vehicles and equipment.  It includes all costs and traffic control required to replace a damaged temporary barrier-mounted sign system as directed by the Engineer.

2.	Conc Barrier-Mounted Temp Sign System, Oper, includes all costs for cleaning, and relocating as necessary the sign system throughout the life of the contract, as well as all costs associated with the repair of barrier sections necessitated by use of this system.

3.	All prismatic signs used in conjunction with this device will be paid for separately with other pay items.