MICHIGAN

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION

FOR

BARRIER GATE REMOVAL, SALVAGE, AND INSTALLATION

BRG:CAW	1 of 2	APPR:DMG:CRB:03-19-24

a.	Description.  This work consists of removing, salvaging, and installing an existing gate assembly and concrete barrier in accordance with sections 204 and 812 of the Standard Specifications for Construction.  The gate could be either a manually operated swing gate or a remote-controlled electrical lift gate.  Remove and salvage the gate and a portion of the concrete barrier to allow temporary rerouting of traffic.  When the traffic shift is no longer needed, ensure the existing gate and concrete barrier are re-installed as previously and functional.

b.	Materials.

1.	Furnish concrete Grade 3500 in accordance with section 1004 of the Standard Specifications for Construction for the barrier and gate foundations.

2.	Furnish all new hardware and accessories in kind necessary to make the gate functional including, but not limited to, gate posts, screws, bolts, nuts, washers, straps and related items in accordance with section 908 of the Standard Specifications for Construction.

c.	Construction.  Remove and salvage the gate assembly, including all associated hardware and operating mechanisms in accordance with subsection 204.03 of the Standard Specifications for Construction.  Replace gates and any hardware damaged during removal at no cost to the contract.

If it is possible to remove the electrical lift gate arm and leave the rest of the electrical gate assembly in place without interference to the temporarily relocated traffic, it is acceptable.  If not, remove and salvage the entire gate assembly.  An opening of at least 43 feet plus 2 feet of shy distance on either side is required to maintain temporary traffic at the existing gate location in accordance with the Maintaining Traffic plan.

If it is possible to leave the foundations for the gate in place without interference to the temporarily relocated traffic, it is acceptable.  If not, replace the foundation in its entirety.

Remove and salvage concrete barrier adjacent to the gate assembly to the limits shown on the plans or as directed by the Engineer.

Temporarily cover and protect all wiring and conduit connected to the gate assembly.

Install concrete barrier in the existing location where it was removed for this work or as directed by the Engineer.

Install the gate assembly (either manual swing or electrical lift) at the existing location shown on the plans or as directed by the Engineer.  Install posts so that the gate is centered on the existing barrier opening just east of the US customs booths.  Install the gate level and spaced to prevent vehicles from passing in the closed position.

When complete, the gate should be fully functional and operate as the current assembly is now.

d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay item:

Pay Item	Pay Unit

Barrier Gate, Rem, Salv, and Install	Each

Barrier Gate, Rem, Salv, and Install includes the removal, salvaging, and installation of an existing gate assembly (either the manually operated swing gate or remote-controlled electrical lift gate).  Unit of payment includes the entire gate assembly consisting of one gate section (either the manually operated swing gate or remote-controlled electrical lift gate) with all hardware, gate posts, concrete foundations and bracing, concrete barrier and protection system, and all other electrical and mechanical materials required to make the gate functional at each location shown on the plans.