MICHIGAN

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION

FOR

BRIDGE BEARING PLATE, CLEAN AND COAT

BRG:ERC	1 of 1	APPR:JAB:SCK:04-10-24

a.	Description.  This work consists of cleaning the sole and masonry plates as called for on the plans or as directed by the Engineer.  Ensure the cleaning is completed in adherence to standard specifications except as modified herein.

b.	Materials.  Furnish ZRC Cold Galvanizing Repair Compound from ZRC Worldwide or approved equal.

c.	Construction.  Clean sole plates and masonry plates in accordance with SSPC SP3, Power Tool Cleaning.  After cleaning is complete notify the Engineer prior to coating.  Coat the cleaned plates in accordance with the manufacturer’s recommendations.  Protect the work and the environment during cleaning.  Protect pedestrian, vehicular, water vessels, and other traffic in accordance with subsection 104.07.B of the Standard Specifications for Construction.

Complete cleaning and coating prior to installing forms or patching materials.

d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay item:

Pay Item	Pay Unit

Bridge Bearing Plate, Clean and Coat	Each

Bridge Bearing Plate, Clean and Coat includes all materials, equipment, and labor necessary to clean and coat the masonry and sole plates.

The Department will pay for each beam ends set of beam plates as a single unit.