MICHIGAN

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION

FOR

BRIFEN TEST LEVEL-4 HIGH-TENSION CABLE BARRIER

GCB:CT	1 of 14	APPR:TEB:RWS:03-21-25

a.	Description.  This work consists of constructing four-cable, Brifen Test Level (TL)-4 high-tension cable barrier (HTCB) and cable terminals, manufactured by Brifen USA, Inc., as shown on the plans.  HTCB systems made by other manufacturers are prohibited when HTCB is measured as specified in this special provision.  Complete this work in accordance with manufacturer’s details and specifications, and this special provision.  If the requirements of this special provision conflict with the requirements of the manufacturer’s details, comply with the requirements of this special provision.

b.	Materials.  Furnish materials in accordance with the manufacturer’s specifications and this special provision.  Furnish written certification to the Engineer stating that the materials used to construct the HTCB system and end terminals meet manufacturer’s specifications and this special provision.

Furnish manufacturer’s written certification to the Engineer that all components supplied by the manufacturer meet manufacturer’s specifications and this special provision.

Furnish all HTCB and end terminals of the same type from Brifen USA, Inc., do not intermix or overlap different HTCB types.  HTCB systems made by other manufacturers are prohibited.  Furnish all line posts of the same type, do not intermix different types of line posts.  Ensure that the end terminals are compatible with the HTCB system installed.

The HTCB must meet or exceed NCHRP Report 350 (NCHRP 350), TL-3 or MASH, TL-3 when installed on a slope with an inclination of 1:4 (1 Vertical on 4 Horizontal).  The HTCB must also meet or exceed NCHRP 350, TL-4 or MASH, TL-4 when installed on a slope with an inclination of 1:6 (1 Vertical on 6 Horizontal) or flatter.  The end terminals must meet or exceed NCHRP 350, TL-3 or MASH, TL-3.  The HTCB and end terminals must have FHWA acceptance.  Furnish FHWA acceptance letters, to the Engineer, stating that the HTCB meets or exceeds NCHRP 350, TL-3 or MASH, TL-3 when installed on a 1:4 slope, the HTCB meets or exceeds NCHRP 350, TL-4 or MASH, TL-4 when installed on a 1:6 slope or flatter, and the end terminals meet or exceed NCHRP 350, TL-3 or MASH, TL-3.

Ensure the HTCB system and all associated components are manufactured in the United States of America and meet Buy America Act requirements.

1.	Cables.  Ensure HTCB has four cables.  Ensure each cable is 3/4-inch (minimum) diameter, 3 × 7 construction, zinc-coated (galvanized) wire rope manufactured in accordance with AASHTO MP30M/MP30, Type I, Class A coating.  Ensure each cable has a minimum tensile strength of 39,000 pounds.  Ensure each cable is factory pre-stretched after manufacture with a tensile load of 50 percent (minimum) of the cable’s tensile strength to prevent future strain relaxation of the cable.  Do not damage the cable during the pre-stretching process.  Ensure each cable has a minimum modulus of elasticity of 11,805,090 psi after pre-stretching.

Furnish documentation with each cable spool, from the cable manufacturer to the Engineer, certifying the breaking strength of the cable, the amount of force used to pre-stretch the cable, the modulus of elasticity of the cable after pre-stretching, and the pre-stretching/testing date(s).

2.	Posts and Fittings.  Ensure all posts are made of steel meeting ASTM A36/A36M.  Ensure all posts are zinc coated (galvanized) after fabrication to ASTM A123/A123M.

Ensure all fittings, including but not limited to turnbuckles and connections, have a minimum diameter of 3/4 inch.  Ensure all fittings develop a minimum tensile load (without yielding) of 36,800 pounds.  Ensure the manufacturer conducts one tensile load test on each fitting type used in the HTCB system.  Furnish documentation, from the manufacturer to the Engineer, certifying that all fitting types have been tested and meet the minimum load requirements specified in this special provision.  Ensure the documentation also lists the tensile yield strength and test date(s) for each fitting type.

Ensure threaded terminals are right hand or left hand threaded M24 × 3 pitch in accordance with ANSI B1.13M.  A maximum of one open-type wedge lock terminal or two closed-type wedge lock terminals will be allowed per cable per run (between end anchor foundations).  Closed-type wedge lock terminals must utilize a threaded-end socket to secure the wedge by compression.  Only one wedge lock terminal type (open-type or closed-type) is permitted.  Ensure all other terminals are of the swaged type.  Swaged type terminals may be shop or field swaged.

Ensure the body of the threaded terminal furnishes a minimum of 5.9 inches wire rope engagement depth.  Ensure fully fitted ropes develop a minimum breaking load of 36,800 pounds.  Ensure threaded terminals are galvanized, after threading, in accordance with ASTM A153/A153M.

Ensure one end of each turnbuckle is threaded right hand and the other end left hand in accordance with ANSI B1.13M, M24 x 3 to accept threaded rope terminals.  Ensure turnbuckles are of the solid or closed body type with two inspection holes to determine threaded rope terminal penetration.  Ensure turnbuckles allow for a minimum of 6 inches of penetration from each end.

Ensure all fittings, including but not limited to turnbuckles and connections, are either zinc coated (galvanized) in accordance with ASTM A153/A153M after threading, or are made of stainless steel.  Ensure all other components made of ferrous metal, excluding stainless steel components, are zinc coated (galvanized) according to ASTM A123/A123M after fabrication.

3.	Reflective Sheeting.  Ensure Type XI reflective sheeting is attached to all reflectors as specified in subsection 919.03.B of the Standard Specifications for Construction.  Ensure reflectors meet manufacturer’s specifications.  Ensure reflectors match the color of edge line adjacent to approaching traffic.  Ensure each reflector has a minimum of 13 square inches of reflective sheeting facing approaching traffic.

4.	Concrete/Foundation Materials.  Furnish Grade 4000 or 4000HP concrete in accordance with section 1004 of the Standard Specifications for Construction for all foundations, except ensure that concrete slump is modified in accordance with note k or l of Table 1004-1 of the Standard Specifications for Construction.

Furnish concrete curing materials in accordance with subsection 903.07 of the Standard Specifications for Construction.

Furnish temporary casing material in accordance with subsection 919.10 of the Standard Specifications for Construction unless otherwise shown on the plans.

Furnish slurry in accordance with subsection 718.03.E of the Standard Specifications for Construction.  Use only polymer type slurries.

5.	Steel Reinforcement.  Furnish epoxy coated steel reinforcement for concrete foundations in accordance with section 905 of the Standard Specifications for Construction.

6.	Miscellaneous Materials.

A.	Ensure excluder caps meet manufacturer’s specifications and are made of low-density polyethylene or polypropylene.

B.	Use sound earth meeting the requirements specified in section 205 of the Standard Specifications for Construction for grading and earthwork.

C.	Use marine-grade anti-seize lubricant for threaded fittings that is acceptable for use on galvanized steel.

D.	Ensure sockets for line post foundations meet manufacturer’s specifications.  In addition, ensure sockets embedded in concrete foundations are fabricated from 11 gauge (minimum), hot rolled mild steel galvanized to ASTM A123/A123M, after fabrication.

c.	Manufacturer’s Representative.  Ensure that prior to HTCB installation, the HTCB manufacturing company furnishes the Engineer with the name, telephone number, electronic mail (e-mail) address, and a resume of a representative from the HTCB system manufacturing company that has been assigned to this project.  Ensure the manufacturer’s representative is employed, either directly or under contract, by the HTCB manufacturer.  The manufacturer’s representative cannot be employed, either directly or under contract, by the Contractor.  The Contractor is prohibited from acting as the manufacturer’s representative.

The manufacturer’s representative must have thorough knowledge of the HTCB system being installed and must have prior experience installing the HTCB system used for this project.  The representative’s resume must specify the length of time working for the manufacturer and contain a list detailing the HTCB projects the representative has worked on over the last 3 years.  The Engineer reserves the right to reject a manufacturer’s representative if the representative fails to demonstrate thorough knowledge of the HTCB system being installed, fails to submit proof of prior experience installing the HTCB system used for this project, or fails to comply with the requirements of this special provision.  If the Engineer rejects a manufacturer’s representative at any time during construction, the HTCB system manufacturing company must provide a different manufacturer’s representative, meeting the requirements of this special provision, within 2 working days and at no additional cost to the contract.

The manufacturer’s representative must respond to any telephone or e-mail inquiries from the Engineer within 2 working days.  If requested by the Engineer, the manufacturer’s representative must travel to the project site and meet with the Engineer to inspect the HTCB installation and discuss any issues regarding the HTCB installation.  The manufacturer’s representative must travel to the project site and meet with the Engineer no later than 5 working days after the Engineer’s request.

No later than 5 days after initial cable tensioning of all HTCB runs, the manufacturer’s representative must travel to the project site, meet with the Engineer, and inspect the entire HTCB installation.  Any deviations from manufacturer’s specifications must be reported directly to the Engineer.

d.	Consultation and Training.  Ensure the HTCB manufacturing company is available to consult with and train personnel from MDOT and/or any of MDOT’s invitees, without additional cost, as requested by MDOT.  Consultation and training must encompass the design, installation, operation, and maintenance of the HTCB system.

The manufacturer must furnish training with respect to the design, installation, operation, and maintenance of the HTCB system.  Ensure training and consultation are held at a location in the State of Michigan deemed acceptable by MDOT.  The manufacturer must issue a dated certificate to each individual that has undergone formal training.  The manufacturer must comply with all of the training requirements specified in the contract.

Prior to installation of the HTCB system, furnish written certification from the manufacturer to the Engineer, that the entire work force to be used for installing the system has received the training and necessary aids to install the system.  This work force training must include installation of foundations, end terminals, posts, cables, turnbuckles, reflectors, miscellaneous hardware, and tensioning of the cables.  The written certification must contain a list of individuals trained and certified by the manufacturer.  Furnish an updated list of workers trained and certified by the manufacturer no later than 48 hours after personnel changes occur.

e.	Plans and Shop Drawings.  At least 30 days prior to HTCB installation, electronically submit a complete set of plans developed by the manufacturer of the HTCB system selected for the project.

The Department will require up to 30 days to review plans and shop drawings.  The Contractor and manufacturer must address any questions, comments, or concerns raised by MDOT personnel.  Do not commence HTCB construction/installation until the plans and shop drawings have been reviewed and approved by the Department.

Incomplete shop drawings will not be accepted or reviewed by the Department.  The Contractor will be held responsible for incomplete and/or late plan set submittals.  Extension of the completion dates for the project will not be granted due to incomplete and/or late plan set submittals.

Ensure all drawings and calculations are in English units.  Ensure the HTCB system is designed in accordance with manufacturer’s recommendations and this special provision.  Ensure shop drawings detailing the end terminal foundations are signed and sealed by a Professional Engineer licensed in the State of Michigan.

Include all of the following in each set of plans:

1.	Detailed shop drawings of the HTCB system, design calculations and notes, and any construction specifications.

2.	One drawing for each HTCB run, containing the following items:

A.	The height of each cable in the system;

B.	The post length and height of each post with respect to ground level;

C.	The post spacing along the entire length of each cable run;

D.	Detailed drawings of all posts and hardware;

E.	Turnbuckle locations;

F.	The overall length of the cable run, including the end terminals;

G.	The HTCB length, excluding the end terminals;

H.	The end terminal design, including end terminal length;

I.	End terminal locations (stations); and

J.	The foundation dimensions and detailed steel reinforcement layout for all concrete foundations in the HTCB run.

3.	Reflector design.

4.	End terminal foundation design(s), signed, and sealed by a Professional Engineer licensed in the State of Michigan, showing foundations dimensions and steel reinforcement layout.

5.	A report detailing the methodology and geotechnical data (including soil boring logs and soil test results) used to design line post foundations and end terminal foundations.  Ensure geotechnical data used to design foundations, supporting calculations, steel reinforcement calculations, and conclusions/recommendations are included and clearly identified in the report.

6.	A table showing the recommended post spacing as a function of roadway curvature.

7.	A table or graph, or both, showing impact deflection (under NCHRP 350, TL-3 or MASH, TL-3 conditions) as a function of post spacing.

8.	A table showing the recommended cable tension as a function of cable temperature.

9.	Detailed drawing of a modified cable post and/or hardware that can accommodate turnbuckles and/or fittings.

10.	A signed certification letter from the manufacturer indicating the HTCB system conforms to this special provision.

11.	FHWA acceptance letter indicating the HTCB meets or exceeds NCHRP 350, TL-3 or MASH, TL-3 when installed on a 1:4 slope.

12.	FHWA acceptance letter indicating the HTCB meets or exceeds NCHRP 350, TL-4 or MASH, TL-4 when installed on a 1:6 slope or flatter.

13.	FHWA acceptance letter indicating the end terminals meet NCHRP 350, TL-3 or MASH, TL-3.

f.	Geotechnical Information.  Limited geotechnical information is furnished by the Department on the project plans.  Geotechnical information furnished by the Department may be used for design purposes at the manufacturer’s discretion.  It is expressly understood that the Department will not be responsible for interpretations or conclusions drawn from geotechnical information furnished by the Department by the Contractor, manufacturer, and/or any of their affiliates.  Soil data furnished by the Department represents conditions at specific locations.  No inference should be made that subsurface conditions are the same at other locations.  Additional soil data will not be furnished by the Department.

Contact the manufacturer prior to bidding and determine if additional geotechnical data is required.  Obtain and furnish additional geotechnical data (including soil borings and soil testing data) and laboratory tests required for the manufacturer to complete design of end terminal foundations or other components of the HTCB system.  Conduct laboratory tests in accordance with AASHTO and ASTM Standard Methods of Testing.

g.	General HTCB System Design.  Design the HTCB system in accordance with the manufacturer’s recommendations and specifications, and the following general requirements.

1.	Ensure the modified cable post and/or hardware for accommodating turnbuckles and/or fittings meets manufacturer’s specifications and must not undermine the crash worthiness of the HTCB system.

2.	Ensure individual cables terminate at an end terminal foundation.  Anchoring individual cables to other cables is prohibited.

3.	Ensure end terminal foundation designs satisfy all the following requirements.

A.	Ensure the minimum design load for end terminal foundations and cable connections to the foundation is based on the theoretical cumulative cable tension expected at ­25 °F.

B.	Ensure each end terminal foundation is designed using the P-Y Method (e.g., L­Pile) when checking the theoretical deflection and must use the Broms’ Design Methods when checking overturning.

C.	Ensure each end terminal foundation is designed using the appropriate geotechnical information furnished by the Department and/or appropriate geotechnical information furnished by the Contractor, as determined by the manufacturer.

D.	Use a minimum factor of safety of 2.5 against overturning using the Broms’ Design Methods (Refer to Subsection 13.6.1.1 of the Standard Specification for Structural Supports for Highway Signs, Luminaires and Traffic Signals, 6th Edition, 2013).

E.	Ensure end terminal foundation deflection is not greater than 0.5 inch when subjected to the minimum end terminal design load described in this special provision.  Use a minimum factor of safety of 1.0 in the foundation deflection analysis.

F.	Ensure end terminal foundations are designed to resist uplift and/or downward forces using a minimum factor of safety of 2.0 and the Beta and Alpha Methods as described in FHWA-IF-99-025 Drilled Shaft:  Construction Procedures and Design Methods (O’Neill and Reese, 1999).

G.	Ensure steel reinforcement for end terminal foundations is designed using appropriate AASHTO guidelines.

H.	Ensure drilled shaft (cylindrically shaped) concrete foundations are used for all end terminal foundations and post foundations.  A rectangular reinforced concrete pile cap may be used to connect two or more cylindrically shaped end terminal foundations.  Reinforced concrete pile caps must have a minimum depth of 48 inches.

h.	Concrete Foundation Construction.  Unless directed otherwise by the Engineer, construct concrete foundations in accordance with the details on the plans, the plan set developed for this project by the manufacturer, section 718 of the Standard Specification for Construction, and this special provision.

Embed a sleeve/socket in the concrete of each line post foundation to hold the post.  Place steel reinforcement in each line post foundation in accordance with the plans, the plans developed by the manufacturer for this project, and this special provision.

Place steel reinforcement in each end terminal foundation in accordance with the plan set developed by the manufacturer for this project and this special provision.

Ensure that the bottoms of all concrete foundations, including but not limited to line post foundations and end terminal foundations, are a minimum of 48 inches below ground level.  Ensure that the tops of all concrete foundations are at ground level.  The minimum diameter for all foundations is 15 inches.  The top of all concrete foundations must have a smooth finish.

Conduct concrete QC and QA in accordance with the contract, or as directed by the Engineer.

•	Qualifications of Drilled Shaft Contractor.  The Contractor constructing cable terminal foundations must have installed drilled shafts with diameters up to 4 feet and depths in excess of 30 feet within a period of 3 years or less prior to the bid date for this project.  The Contractor must demonstrate to the Engineer that the Contractor's supervisor and drillers performing the work have completed at least three projects of similar scope, drilled shaft diameters and depths, and subsurface conditions to this project.  The Contractor's supervisor must have at least 3 years of acceptable experience in installing similar types of drilled shafts.

•	Drilled Shaft Installation Plan.  In addition to the requirements of subsection 718.03.A of the Standard Specifications for Construction, the drilled shaft installation plan must include the methods proposed to prevent drilled shaft excavation spoils from entering the waters of the state or stormwater drainage systems, when the work is within the influence of same.

•	Subsurface Data.  Review the available geotechnical information for this project.  Notify the Engineer in writing within 48 hours of determining that the actual subsurface conditions differ substantially from those reported on the boring logs.  It is expressly understood that the Department will not be responsible for interpretations or conclusions drawn by the Contractor from geotechnical information provided by the Department.

4.	Casing.  Determine the need for casing the shaft excavation and if casing is required, use shaft excavation casing temporarily, and remove casing in accordance with this special provision.  The use of permanent shaft excavation casing is prohibited.

5.	Protection of Existing Structures, Utilities, Culverts, and Drain Pipes.  Control operations to prevent damage to existing structures, utilities, culverts, and drain pipes.  Preventative measures include, but are not limited to, selecting construction methods and procedures that will prevent caving of the shaft excavation, monitoring and controlling the vibrations from construction activities (such as installation of casing and drilling of the shaft), and monitoring and controlling the depth of excavation.  Repair any damage to existing structures, utilities, culverts, or drain pipes, to the satisfaction of the Engineer at no additional cost to the contract, including engineering analysis and redesign, and without any extension of the completion dates for the project.

6.	Construction Tolerances.  The manufacturer of the HTCB system selected for this project must prepare a complete list of construction tolerances, including but not limited to cable height, post spacing, and horizontal alignment, for the HTCB system selected for this project.  Submit the list of construction tolerances to the Engineer for review and approval.  Obtain the Engineer’s approval of all construction tolerances prior to beginning HTCB construction/installation.

Manufacturer’s tolerances must comply with the following minimum construction tolerances:

A.	Ensure at the drilled shaft actual bottom elevation, the out-of plumb is no greater than one percent of the drilled shaft length as measured from the actual center of the shaft at the shaft design top elevation.

B.	Ensure after HTCB installation, the out-of-plumb of all HTCB line posts is no greater than one percent in all directions.  The use of plastic shims for leveling line posts is prohibited.

C.	Ensure after all the shaft concrete is placed, the top of the reinforcing steel cage for end terminal foundations and line post foundations is no more than 1 inch above or below plan position.

D.	The top elevation of all concrete foundations must have a tolerance of 0 to 1 inch above the top of shaft elevation.  Ensure the top of all concrete foundations is not below ground level.

Drilled shaft excavations and completed foundations not constructed and installed within the required tolerances will be considered unacceptable.  Correct all unacceptable shaft excavations and completed foundations to the satisfaction of the Engineer.  Furnish labor, equipment, and materials necessary to complete corrections for out of tolerance drilled shafts and posts, including engineering analysis and redesign, at no additional cost to the contract, and without an extension of the completion dates for the project.

7.	Excavation.  Maintain the stability of the excavation sidewalls and extend the shaft excavation to a stratum approved by the Engineer.  Extend drilled shaft tip elevations when the Engineer determines the bearing stratum encountered during excavation is unsuitable or differs from that anticipated in the design of the drilled shaft.  Furnish the Engineer access to auger cuttings of the bearing material for additional analysis.  Fill over-excavation of shafts, and unauthorized shaft excavations extended below required depths or elevations, with concrete when constructing concrete foundations, at no additional cost to the contract.

Do not leave an uncased drilled shaft excavation open overnight.  If, when constructing concrete foundations, an uncased drilled shaft cannot be completed in one day, backfill the excavation with flowable fill.

Dispose of excavated materials removed from shaft excavations, and not used for slope grading, in accordance with subsection 205.03.P of the Standard Specifications for Construction.  Keep excavated materials away from each open shaft excavation.  Remove excavated materials immediately after completing the shaft excavation, or as directed by the Engineer.  Direct surface water away from shaft excavations.  Ensure that excavated material does not enter the waters of the state or stormwater drainage systems.

8.	Obstructions.  Remove surface and subsurface obstructions encountered in the length of excavation at concrete foundation locations.  Such obstructions may include materials such as old concrete foundations or abandoned utilities, or natural materials such as boulders.  In the event that the excavation cannot be advanced using conventional augers fitted with soil or rock teeth, drilling buckets and/or under reaming tools, use special procedures or tools including, but not limited to, chisels, boulder breakers, core barrels, air tools, hand excavation, and enlarging the hole diameter.  Unless otherwise specified in the contract, removal of such obstructions will be paid for as extra work provided that special procedures or tools are utilized.  To be considered for payment for obstruction removal, submit written notification to the Engineer no later than 24 hours after encountering obstructions and allow the Engineer to inspect the excavation and verify that special procedures or tools are required prior to undertaking the removal.

9.	Excavation Inspections.  Ensure excavation inspection is in accordance with subsection 718.03.F.1 of the Standard Specifications for Construction, with the following addition:  after shaft excavation, and provided that a tremie pour is not required, furnish access and allow time for the Engineer to inspect the shaft.  Furnish suitable lighting if needed.

10.	Steel Reinforcement.  Place steel reinforcement in concrete foundations as specified on the plans and shop drawings developed for the project.  Construct and place reinforcing steel cage in accordance with subsection 706.03.E of the Standard Specifications for Construction and the following:

A.	Completely assemble a reinforcing cage, consisting of longitudinal bars, stiffener bars, centralizers and tie reinforcement or spiral reinforcement prior to placement in the shaft excavation.  Tie together steel reinforcement cages for all HTCB foundations.  Do not use welded steel reinforcement cages.

B.	Place the steel reinforcement as a unit immediately after the shaft excavation is inspected and approved, and prior to concrete placement.

C.	Use non-corrosive spacers on the exterior of the reinforcing cage, near the bottom of the cage and at sufficient intervals to ensure concentric spacing of the cage for its entire length within the shaft excavation.  Ensure the spacers are of adequate dimensions to furnish a minimum 3 inches annular space between the outside of the reinforcing cage and the perimeter of the excavated shaft.

D.	Maintain the bottom of the cage the proper distance above the shaft base using approved concrete bottom supports, or by other methods approved by the Engineer.

E.	Tie and support the reinforcing steel in the shaft during concrete placement and temporary casing removal such that the reinforcing steel will remain within the allowable tolerances.

F.	Check the elevation of the top of the steel cage before and after the concrete is placed.  If the reinforcing steel cage is not maintained within the specified tolerances, make corrections to the satisfaction of the Engineer.  Modify the steel cage support in a manner satisfactory to the Engineer prior to constructing additional shafts.

G.	Use epoxy coated steel reinforcement for all HTCB foundations, including line post foundations.

11.	Concrete Placement.  Complete concrete placement for concrete foundations in accordance with the applicable portions of section 706 and subsection 718.03.H of the Standard Specifications for Construction, and as modified herein.

Ensure that concrete is delivered to the site from the batch plant in a continuous manner to help avoid interruption of placement.  Place concrete, either by free-fall or through a tremie or concrete pump, the same day the shaft is excavated.

Place concrete by free-fall methods only in dry excavations where free water accumulation of 3 inches or less can be maintained immediately prior to concrete placement.  Direct concrete placed by free-fall methods in the center of the shaft to avoid contact with the reinforcing steel cage, shaft sidewalls and temporary casing.  Drop chutes may be used to direct concrete in a vertical stream down the shaft.

Consolidate the concrete in all HTCB foundations, including line post foundations, with vibrators in accordance with the requirements of subsection 706.03.H.1 of the Standard Specifications for Construction.

12.	Temporary Casing.  Coordinate temporary casing withdrawal carefully with concrete placement.  When temporary casing is being withdrawn, maintain a sufficient head of concrete above the bottom of the casing to prevent reduction in the shaft diameter due to earth and/or hydrostatic pressure on the fresh concrete, and to prevent extraneous material from mixing with fresh concrete.  Check the concrete level in the temporary casing prior to, and after casing withdrawal to confirm that separation of the shaft concrete has not occurred.

13.	Protection of Concrete.  Protect fresh concrete from flowing water and damage from mechanical equipment and nearby construction vibrations.  Do not generate vibrations from pounding of sheet piling, pile driving, or casing installation within a radius of 25 feet until the concrete has attained 75 percent of its specified minimum strength.  Protect concrete from strength reduction caused by frost or freezing actions.

Comply with the curing requirements of subsection 810.03.J.8 of the Standard Specifications for Construction for all HTCB foundations, including line post foundations.

i.	HTCB Construction/Installation.  Furnish and install the HTCB system in accordance with the following:

1.	General.  Furnish and install Brifen TL-4 HTCB and end terminals at the location(s) specified on the plans.  Install all HTCB and end terminals in accordance with the plans, this special provision, and the shop drawings developed by the manufacturer for this project.

2.	Posts.  Base post spacing on manufacturer’s specifications depending on the roadway curvature shown on the plans while satisfying the following condition: The post spacing must not exceed 10 feet, 6 inches, unless otherwise specified on the plans developed by the Department or directed by the Engineer.

Install a modified cable post and/or hardware, in accordance with the plans developed by the manufacturer for this project and this special provision, at all locations where a standard cable post cannot be properly installed and attached to the cables due to the presence of a turnbuckle or fitting.

Install excluder caps on all posts placed in sockets, to prevent debris from entering the socket.

3.	Reflectors.  Attach reflectors to line posts in accordance with manufacturer’s specifications and this special provision.  Install reflectors on both sides of the line post, unless otherwise specified on the plans developed by the Department.  Space reflectors at the following intervals:

A.	48 feet (maximum) on tangent sections and curves with a radius of 1,150 feet or greater.

B.	24 feet (maximum) on curves with a radius less than 1,150 feet.

4.	Slope Grading.  Grade around all concrete foundations, as necessary, to remove any ridges, dips, holes, or voids around the foundations and match the surrounding slopes.  Graded areas must have a slope of 1:4 or flatter.  Graded areas must meet Class A slope tolerances in accordance with subsection 205.03.N of the Standard Specifications for Construction.

5.	Fittings.  Ensure that wedge-lock type fittings have a minimum of one wire crimped over the base of the wedge to hold it firmly in place.  The crimped section of wire over the base of the wedge must have a minimum length of 3/16 inch.  Lubricate all threaded fittings with marine-grade anti-seize lubricant prior to assembly and installation.

6.	Cable Tensioning.  Do not tension cables until the concrete in the HTCB foundations has reached a minimum compressive strength of 3,000 psi.

Upon complete assembly of the HTCB, set each cable to the initial tension specified by the manufacturer.  Measure the temperature of each cable prior to tensioning and use this temperature to determine the required tension.  Perform final cable tensioning in each cable a minimum of 2 weeks after initial cable tensioning and, if necessary, adjust the tension to the proper setting.  Submit written certification to the Engineer indicating the date of initial cable tensioning, date of final cable tensioning, the ambient temperature and cable temperature on each of these dates, and the tension in each cable on each of these dates.

Upon complete assembly of the HTCB, ensure that all threaded terminals, except one terminal per cable per run, penetrate a minimum of 3 inches and a maximum of 4 inches into the turnbuckle measured from the ends of the turnbuckle.  Ensure that all threaded terminals, including terminals not required to meet the 3-inch minimum and 4-inch maximum penetration requirements, penetrate the turnbuckle past the inspection holes located on the turnbuckle.  Ensure that the terminal threads are visible through the inspection holes located on the turnbuckle.

Upon completion of the construction/installation of the HTCB, both the Contractor and the manufacturer’s representative must furnish written certifications to the Engineer indicating that the HTCB system and end terminals were installed in accordance with the plans, manufacturer’s specifications and guidelines, and this special provision.

j.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay items:

Pay Item	Pay Unit

Cable Barrier, High Tension, 4-Cable, Brifen Test Level-4	Foot

Cable Barrier Terminal, High Tension, 4-Cable, Brifen Test Level-4	Each

1.	Cable Barrier, High Tension, 4-Cable, Brifen Test Level-4.  The plan quantity for Cable Barrier, High Tension, 4-Cable, Brifen Test Level-4 is based on the overall dimension between the final concrete foundations in each run of HTCB.  Actual final payment will be based on the field measurements deducting the terminal system per the manufacturer's approved shop drawings.  For bidding and ordering purposes, it is the Contractor’s responsibility to make necessary adjustments by deducting the length of the terminal endings from the plan length.

Cable Barrier, High Tension, 4-Cable, Brifen Test Level-4 includes all materials, labor, and equipment required for the following:

A.	Constructing four-cable, HTCB manufactured by Brifen USA, Inc. that complies with the requirements of this special provision;

B.	Constructing the HTCB in accordance with the plans, the plan set developed by the manufacturer for this project, and this special provision;

C.	Obtaining and furnishing soil properties (including soil borings and laboratory testing), as specified by the manufacturer;

D.	Furnishing and installing reflectors, as specified in this special provision;

E.	Constructing line post foundations, as specified on the plans developed by the manufacturer for this project and this special provision;

F.	Conducting concrete QC and QA;

G.	Furnishing a trained work force and a manufacturer’s representative on-site during installation and during consultation/training requested by MDOT;

H.	Preparing and submitting plans and/or requested information to MDOT;

I.	Grading around HTCB foundations;

J.	Removal and disposal of excavated materials;

K.	Furnishing, installing, and removing shaft excavation casings;

L.	Furnishing training/consultation by the manufacturer as requested by MDOT; and

M.	Furnishing a manufacturer’s representative on-call throughout the duration of the project, on-site as requested by the Engineer, and as specified in this special provision, and during consultation/training requested by MDOT.

2.	Cable Barrier Terminal, High Tension, 4-Cable, Brifen Test Level-4 includes all materials, labor, and equipment required for the following:

A.	Constructing a NCHRP 350, TL-3 or MASH, TL-3 compliant HTCB terminal manufactured by Brifen USA, Inc. that is compatible with the HTCB system installed;

B.	Constructing end terminal in accordance with the plans, the plans developed by the manufacturer for this project, and this special provision;

C.	Constructing all end terminal foundations and post foundations, as specified in the plans developed by the manufacturer for this project and this special provision;

D.	Conducting concrete QC and QA;

E.	Obtaining and furnishing soil properties (including soil borings and laboratory testing), as specified by the manufacturer;

F.	Preparing and submitting plans and requested information to MDOT pertaining to the end terminals;

G.	Certification of the manufacturer’s end terminal foundation design(s) by a Professional Engineer licensed in the State of Michigan;

H.	Grading around cable terminal foundations;

I.	Removal and disposal of excavated materials; and

J.	Furnishing, installing, and removing shaft excavation casings.

3.	Payment Schedule.  Payment will be made after final cable tensioning is completed, unless otherwise authorized by the Engineer.

4.	Bid Preparation.  Bidders are required to consult with Brifen USA, Inc. and geotechnical consultants prior to bidding, and estimate foundation dimensions and steel reinforcement quantities based on the foundation design requirements of this special provision and appropriate soil data furnished on the project plans.  Base unit prices on conservative foundation designs that meet or exceed the requirements of this special provision.  Additional payment will not be provided for constructing HTCB foundations with dimensions and/or depths different from those used by the bidder for bidding and estimating purposes.