MICHIGAN

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION

FOR

DITCH LINER

MET:RDM	1 of 2	APPR:DMG:RWS:05-06-21

a.	Description.  This work consists of furnishing and installing a prefabricated ditch liner in ditch bottoms and spillways at the locations shown on the plans.  Complete all work in accordance with the standard specifications, the manufacturer’s installation guidelines and as specified herein.

b.	Materials.  Provide tied concrete block mat in rolls of the specified width.  Ensure a double net straw mulch blanket is adhered to the backside of the ditch liner.

Use one of the items listed below, or an approved equal.

Flexamat by:	ShoreFlex by:

Motz Enterprises, Inc.	Premier Concrete Products Inc.

3153 Madison Road	38200 LA Hwy 16

Cincinnati, OH 45209	Denham Springs, LA 70706

(513) 772-6689	(225) 667-4545

https://www.flexamat.com/	https://www.shoreflex.com/

Provide a General Certification in accordance with the MQAP Manual for the prefabricated ditch liner for acceptance.

c.	Construction.  Prior to placing the ditch liner, prepare the base as shown on the plans and in accordance with the standard specifications.  Clear the area of debris and stones two inches or larger.  Install ditch liner to the line and grade shown on the plans.  Dewater the installation area as necessary to facilitate construction of the ditch liner.

Install and anchor the prefabricated tied concrete block mat in accordance with the manufacturer’s installation guidelines.  The manufacturer must provide on-site technical assistance during the ditch liner or spillway grade preparation and installation as needed or when requested by the Engineer.

When placed at a concrete spillway outlet, provide a minimum 6 inch overlap for the ditch liner and spillway.  Install the ditch liner monolithically with the concrete spillway as shown on the plans.

d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay item:

Pay Item	Pay Unit

Ditch Liner, 4 foot Width	Square Yard

Ditch Liner includes dewatering necessary to complete the work as described.  Concrete and steel reinforcement in the ditch liner and concrete spillway overlap will be included in payment for Spillway, Conc.