MICHIGAN
DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION
FOR

# PART WIDTH STORM SEWER CROSS LEAD CONSTRUCTION

TRV:LCP	1 of 1	APPR:DMG:RPB:08-05-24

a.	Description.  This work consists of part width construction of the trunk line storm sewer cross leads.

b.	Materials.  Furnish materials in accordance with the standard specifications and as specified herein.

Furnish temporary storm sewer pipe in accordance with sections 402 and 909 of the Standard Specifications for Construction as approved by the Engineer.

Furnish geotextile blanket in accordance with section 910 of the Standard Specifications for Construction.

Furnish mortar in accordance with section 1005 of the Standard Specifications for Construction.

c.	Construction.  Ensure part width construction of the storm sewer cross leads are as shown on the construction staging plans and as directed by the Engineer.  Ensure construction of the storm sewer and the design and installation of the steel sheet piling in accordance with sections 402, 403 and 704 of the Standard Specifications for Construction, except as described herein.

Verify invert elevations of existing cross leads and determine approximate elevation of cross leads near centerline prior to placing catch basins and manholes.

Ensure all permanent and temporary sewer joints are wrapped with geotextile blanket.  Seal temporary joints with mortar as directed by the Engineer.  Ensure bedding and subgrade material displaced in stage 2 and stage 3 is replaced.  Backfill trenches daily.

d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay item:

Pay Item	Pay Unit

Part Width Storm Sewer Cross Lead Construction	Each

Part Width Storm Sewer Cross Lead Construction includes dewatering, plating structures, furnishing, placing, and removing steel sheet piling and temporary pipe, cutting existing storm sewer, and wrapping and sealing sewer joints.

Proposed storm sewer structures, covers and permanent pipe will be paid for separately.