MICHIGAN

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION

FOR

SEWER PIPE COUPLING DEVICE

BCY:DJM	1 of 1	APPR:DMG:CJD:05-20-24

a.	Description.  This work consists of connecting new section(s) of sewer pipe utilizing a mechanical coupling to existing sewer pipe, following the removal of existing drainage structures.  Ensure all work is in accordance with the standard specifications.

b.	Materials.  Furnish sewer coupling products appropriate for the pipe sizes shown on the plans.  Ensure the couplings are manufactured in accordance with ASTM C1173 and rated for underground applications.  Ensure the couplings are shielded and manufactured with a molded one-piece elastomeric sealing gasket.  Ensure the shield, connecting bands, worm drive clamps, nuts, and bolts are stainless steel as recommended by the manufacturer.

Furnish the couplings from one of the following manufacturers or Engineer approved equivalent:

Dallas Specialty & Manufacturing Company

(800) 222-5644

https://dallasspecialty.com

Fernco Incorporated

(810) 503-9000

https://www.fernco.com

Mission Rubber Company

(800) 854-9991

https://www.missionrubber.com

Ensure coupling materials are in compliance with Buy America requirements.

c.	Construction.  Connect the proposed pipe to the existing pipe with one of the approved pipe coupling devices.  Ensure the replacement pipe is of the same size and material of the removed pipe.  Install the pipe coupling devices in accordance with the manufacturer’s recommended installation procedure.

d.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay item:

Pay Item	Pay Unit

Sewer Pipe Coupling Device	Each

Sewer Pipe Coupling Device includes all work to furnish and install the approved device in accordance with the manufacturer’s recommended installation procedure.