MICHIGAN
DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION
FOR

# STEEL TRASH RACK

GLD:MRR	1 of 2	APPR:MJF:DMG:02-12-24

a.	Description.  This work consists of furnishing and installing a galvanized steel trash rack including plates and miscellaneous metal hardware as shown on the plans.  Complete this work in accordance with the standard specifications and this special provision.

b.	Materials.  Furnish materials in accordance with the following sections of the Standard Specifications for Construction.

Adhesive Anchoring	712

Structural Steel	906

Anchor Bolts, Nuts and Washers	908

Miscellaneous Metal Products	908

Structure Expansion Anchors and Bolts	914

Furnish steel for trash racks in accordance with ASTM A709/A709M Grade 36 and section 906 of the Standard Specifications for Construction unless otherwise noted in the contract.

Hot dip galvanize trash racks after fabrication in accordance with to ASTM A123/A123M and ASTM A153/A153M where applicable.

Fabricate trash racks in accordance with the dimensions shown on the plans.

c.	Shop Drawing Submittals.  Prepare shop drawings of fabrication details in accordance with subsection 104.02 of the Standard Specifications for Construction.  Do not use contract plans instead of shop drawings.  Clearly indicate any field measurements that differ from the plans or the shop drawings.  Do not start fabrication before the Engineer approves the shop drawings.  Furnish the QAI with a hardcopy of the approved shop drawings.

The approval of shop drawings does not relieve the Contractor from the responsibility to correct errors or omissions or to furnish adequate field measurements as may be required.  It is the Contractors responsibility to call attention to all deviations from the plans, specifications, and details.  If deviations have not been clearly identified, they will not be considered as part of the shop drawing approval.

d.	Construction.  Prepare existing opening to receive new trash rack.  Fabricate trash rack sections as shown on the plans to fit existing openings.  Secure trash racks as shown on plans.

Repair all galvanized areas damaged during construction in accordance with subsection 716.03.E of the Standard Specifications for Construction.  If damage is excessive, the Engineer reserves the right to require the entire trash rack to be hot dip galvanized again at no cost to the contract.

e.	Measurement and Payment.  The completed work, as described, will be measured and paid for at the contract unit price using the following pay item:

Pay Item	Pay Unit

Steel Trash Rack	Each